Ground-Roll Attenuation using a unsupervised ResUNet approach

Yago Salerno Dayub Campos, Gilson Alexandre Ostwald Pedo da Costa, Guilherme Lucio Abelha Mota Authors, UERJ

Copyright 2023, SBGf - Sociedade Brasileira de Geofísica.

This paper was prepared for presentation during the 18th International Congress of the Brazilian Geophysical Society held in Rio de Janeiro, Brazil, 16-19 October 2023.

Contents of this paper were reviewed by the Technical Committee of the 18th International Congress of the Brazilian Geophysical Society and do not necessarily represent any position of the SBGf, its officers or members. Electronic reproduction or storage of any part of this paper for commercial purposes without the written consent of the Brazilian Geophysical Society is prohibited.

_____________________________________________________________________________________________________________________________________________________

# Abstract

Ground-Roll is a coherent noise found in terrestrial seismic surveys. Removing them from the seismogram is not always a simple task, as it masks the entire signal where it is located. In addition to the difficulty of obtaining reference data from an attenuated seismogram, the proposed method for ground-roll attenuation is an unsupervised deep learning model which consists of using a convolutional neural network with the structure of Encoder-2Decoder. The seismic data will be used as input for the Encoder extracting a feature map, where it will be used in the two Decoders in order to transform it into an output containing signal and the other noise. With the aid of a frequency contrast coupled to the loss function, the separation of signal and noise can be performed using the patterns of the frequency spectrum associated with the ground-roll.